# Using the DAC on ConnectBOB mezzanine board of ZynqBC

Overview

This guide will provide a step-by-step walk-through of creating a hardware design using the Vivado IP Integrator for the zynqBC and running a BME280 sensor.

At the end of this tutorial, you will have:

- Created a hardware design for BME sensor.

- Created a .C project in XIlinx Vivado SDK and running the BME functionality using the given functions in C

Prerequisites

Hardware

- zynqBC Development Board and JTAG programmer for programming ZYNQ board

Software

- Xilinx Vivado 2020.2 with the vitis.

Board Support Files

- zynqBC Support Files

  - Follow this link to download the board files from the internet and copy them in the board files folder i.e.

    - C:\Xilinx\Vivado\2020.2\data\boards\board_files

or

    - E:\Xilinx\Vivado\2020.2\data\boards\board_files

1. Creating a New Project

When you first run Vivado this will be the main start window where you can create a new project or open a recent one.

  - Click on Create New Project.

  - You will be presented with the project creation wizard. Click Next.

  - Enter a project name and location the click Next.

  - Select RTL Project and click Next.

  - This demo does not use any existing sources, existing IP or constraints. Click through the next three screens.

  - Select Boards and select the zynqBC board file. Click Next and then Finish. Just make sure before this step that board files are placed already.

- Creating a New Block Design

  - Once the process has completed, click Create Block Design in the flow navigator.

  - Click Ok

  - A blank Block Design will open up.

- Add the Zynq IP & SPI Blocks

  - Click the  Add IP button and search for ZYNQ. Double click on ZYNQ 7 Processing System to place the bare Zynq block.

  - Click the Run Block Automation link

Your Zynq block should now look like the picture below.

  - Click on ADD IP button and search for axi_quad_spi to place it in design. In SPI block expand SPI_0. The block diagram look like this.

  - In this SPI block right click on io0_o and click on make it external. Similarly do this for io1_i, sck_o and ss_o[0:0]. Unexpand SPI_0, block diagram look like this.

Now click on Run Connection Automation again and make sure to check All automation box. Press OK

  - Your new block design must look like this

- Generate HDL Wrapper and Validate Design

  - Select  Validate Design. This will check for design and connection errors.

  - After the design validation step, we will proceed with creating a HDL System Wrapper. In the block design window, under the Design Sources tab, right-click on the block diagram file. We labelled it “design_1.bd” and select Create HDL Wrapper.

- This will create a top module in Verilog and will allow you to generate a bitstream. But before that we need to set the XDC file for the GPIO which is added for LDAC

- Generate Synthesis

  - Click on Run Synthesis

  - Once Completed, a message box will appear. Check View Report and Press OK

  - Click on Constraints Wizards under Synthesis

  - Pop up will appear with No Constraints file. Click Define Target   Create File and give top as file name

  - Check the top.xdc block and Press OK

  - Under IO Ports, select the pin for SPI. The voltage standard will be LVCMOS33 and pins configuration is shown in figure below.

  - Press CTRL+S to save the xdc file settings and press Ok

- Generate the Bitstream

  - Click on Generate Bitstream at the bottom of the Flow Navigator. Wait for the process to complete and click OK.

- Export hardware files for SDK

  - Go to file→Export→Export Hardware… Make sure to check the box for Include bitstream then click OK.

- Launch Vitis Ide

  - Once bitstream is generated, Go to Tools→Launch Vitis IDE and click OK.

  - Set your worksapce. And click on Launch.

- Create an Empty Hello World Application on Vitis Ide

  - Click on File and Create an Application Project. Click on next.

  - Go to create a new platform hardware(XSA) tab and browse for your design file. Then click Next.

  - Give the name of your project and click Next.

  - Again click Next.

  - Select Hello World and click Finish.

  - Copy the following code in helloworld.c

#include <stdio.h>

#include "platform.h"

#include "xil_printf.h"

#include "xgpio.h"

#include "xstatus.h"

#include "xspi.h"

#include "Xil_io.h"

XSpi_Config *ConfigPtr;	/* Pointer to Configuration data */

XSpi Spi;

int spiInit(void);

//function to read from 9dof accelerometer/gyro registers using spi

void dof_ag_Read(u8 Add, u8 * buffer, u8 N){

u8 Write[N+1];

Write[0] = Add | 0x80;			//R/W = 1

XSpi_Transfer(&Spi, Write, buffer, N+1);

}

//function to write in 9dof accelerometer/gyro registers using spi

void dof_ag_Write(u8 Add, u8 val){

u8 Write[2], Read[2];

Write[0] = Add & 0x7F;		//R/W = 0

Write[1] = val;

XSpi_Transfer(&Spi, Write, Read, 2);

}

void ag_check( void){

u8 readbuffcheck[2];

dof_ag_Read(0x0F,readbuffcheck,2);

if (readbuffcheck[1]==0x68){

print("AG Spi is working\n\r");

}

else {

print("AG Spi is not working\n\r");

}

}

void initGyro(){

dof_ag_Write(0x10,0xC0);

dof_ag_Write(0x11,0x00);

dof_ag_Write(0x12,0x00);

dof_ag_Write(0x1E,0x38);

}

void initAccel(){

dof_ag_Write(0x1F,0x38);

dof_ag_Write(0x20,0x00);

dof_ag_Write(0x21,0x00);

}

float get_acceleration(int16_t* axaxis,int16_t* ayaxis,int16_t* azaxis){

int16_t ax1,ax2,ay1,ay2,az1,az2;

u8 accelerometerbuff[10]={0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00};

//dof_ag_Write(0x20,0x48);   //setting accelerometer settings

dof_ag_Read(0x27,accelerometerbuff,7);   //reading accelerometer values from register

ax1=(int16_t)accelerometerbuff[2];

ax2=(int16_t)accelerometerbuff[3];

ay1=(int16_t)accelerometerbuff[4];

ay2=(int16_t)accelerometerbuff[5];

az1=(int16_t)accelerometerbuff[6];

az2=(int16_t)accelerometerbuff[7];

axaxis=(ax2<<8)|ax1;

ayaxis=(ay2<<8)|ay1;

azaxis=(az2<<8)|az1;

//return xaxis,yaxis,zaxis;

}

float get_gyro(){

int16_t gx1,gx2,gy1,gy2,gz1,gz2;

u8 gyrobuff[10]={0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00};

//dof_ag_Write(0x10,0x20);   //setting gyro

dof_ag_Read(0x17,gyrobuff,7);  //reading gyro values from register

gx1=(int16_t)gyrobuff[2];

gx2=(int16_t)gyrobuff[3];

gy1=(int16_t)gyrobuff[4];

gy2=(int16_t)gyrobuff[5];

gz1=(int16_t)gyrobuff[6];

gz2=(int16_t)gyrobuff[7];

int16_t gxaxis=(gx2<<8)|gx1;

int16_t gyaxis=(gy2<<8)|gy1;

int16_t gzaxis=(gz2<<8)|gz1;

}

int main()

{

init_platform();

spiInit();

ag_check();

initAccel();

initGyro();

while(1){

//    	u8 accelbuff[10]={0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00};

//    	dof_ag_Write(0x20,0x48);

//    	dof_ag_Read(0x28,accelbuff,6);

int16_t axaxis,ayaxis,azaxis,gxaxis,gyaxis,gzaxis;

get_acceleration(&axaxis,&ayaxis,&azaxis);

//for(int i=0; i<10;i++){};

get_gyro();

//for(int i=0; i<10;i++){};

}

cleanup_platform();

return 0;

}

int spiInit(void){

int Status;

ConfigPtr = XSpi_LookupConfig(0U);

if (ConfigPtr == NULL) {

return XST_DEVICE_NOT_FOUND;

}

Status = XSpi_CfgInitialize(&Spi, ConfigPtr,ConfigPtr->BaseAddress);

if (Status != XST_SUCCESS) {

return XST_FAILURE;

}

Status = XSpi_SetOptions(&Spi, XSP_MASTER_OPTION );

if (Status != XST_SUCCESS) {

return XST_FAILURE;

}

XSpi_Start(&Spi);

XSpi_IntrGlobalDisable(&Spi);

XSpi_SetSlaveSelect(&Spi, 0x01);

}