# Using the DAC on ConnectBOB mezzanine board of ZynqBC

Overview

This guide will provide a step-by-step walk-through of creating a hardware design using the Vivado IP Integrator for the zynqBC and running a BME280 sensor.

At the end of this tutorial, you will have:

- Created a hardware design for BME sensor.

- Created a .C project in XIlinx Vivado SDK and running the BME functionality using the given functions in C

Prerequisites

Hardware

- zynqBC Development Board and JTAG programmer for programming ZYNQ board

Software

- Xilinx Vivado 2020.2 with the vitis.

Board Support Files

- zynqBC Support Files

  - Follow this link to download the board files from the internet and copy them in the board files folder i.e.

    - C:\Xilinx\Vivado\2020.2\data\boards\board_files

or

    - E:\Xilinx\Vivado\2020.2\data\boards\board_files

1. Creating a New Project

When you first run Vivado this will be the main start window where you can create a new project or open a recent one.

  - Click on Create New Project.

  - You will be presented with the project creation wizard. Click Next.

  - Enter a project name and location the click Next.

  - Select RTL Project and click Next.

  - This demo does not use any existing sources, existing IP or constraints. Click through the next three screens.

  - Select Boards and select the zynqBC board file. Click Next and then Finish. Just make sure before this step that board files are placed already.

- Creating a New Block Design

  - Once the process has completed, click Create Block Design in the flow navigator.

  - Click Ok

  - A blank Block Design will open up.

- Add the Zynq IP & SPI Blocks

  - Click the  Add IP button and search for ZYNQ. Double click on ZYNQ 7 Processing System to place the bare Zynq block.

  - Click the Run Block Automation link

Your Zynq block should now look like the picture below.

  - Click on ADD IP button and search for axi_quad_spi to place it in design. In SPI block expand SPI_0. The block diagram look like this.

  - In this SPI block right click on io0_o and click on make it external. Similarly do this for io1_i, sck_o and ss_o[0:0]. Unexpand SPI_0, block diagram look like this.

Now click on Run Connection Automation again and make sure to check All automation box. Press OK

  - Your new block design must look like this

- Generate HDL Wrapper and Validate Design

  - Select  Validate Design. This will check for design and connection errors.

  - After the design validation step, we will proceed with creating a HDL System Wrapper. In the block design window, under the Design Sources tab, right-click on the block diagram file. We labelled it “design_1.bd” and select Create HDL Wrapper.

- This will create a top module in Verilog and will allow you to generate a bitstream. But before that we need to set the XDC file for the GPIO which is added for LDAC

- Generate Synthesis

  - Click on Run Synthesis

  - Once Completed, a message box will appear. Check View Report and Press OK

  - Click on Constraints Wizards under Synthesis

  - Pop up will appear with No Constraints file. Click Define Target   Create File and give top as file name

  - Check the top.xdc block and Press OK

  - Under IO Ports, select the pin for SPI. The voltage standard will be LVCMOS33 and pins configuration is shown in figure below.

  - Press CTRL+S to save the xdc file settings and press Ok

- Generate the Bitstream

  - Click on Generate Bitstream at the bottom of the Flow Navigator. Wait for the process to complete and click OK.

- Export hardware files for SDK

  - Go to file→Export→Export Hardware… Make sure to check the box for Include bitstream then click OK.

- Launch Vitis Ide

  - Once bitstream is generated, Go to Tools→Launch Vitis IDE and click OK.

  - Set your worksapce. And click on Launch.

- Create an Empty Hello World Application on Vitis Ide

  - Click on File and Create an Application Project. Click on next.

  - Go to create a new platform hardware(XSA) tab and browse for your design file. Then click Next.

  - Give the name of your project and click Next.

  - Again click Next.

  - Select Hello World and click Finish.

  - Copy the following code in helloworld.c

#include <stdio.h>

#include "platform.h"

#include "xil_printf.h"

#include "xstatus.h"

#include "xspi.h"

#include "Xil_io.h"

#include <math.h>

XSpi_Config *ConfigPtr;	/* Pointer to Configuration data */

XSpi Spi;

float readbmetemp();

float readbmepress();

float readbmehumid();

void BMEconfig();

void readBuff();

int spiInit(void);

#define BME280_REGISTER_DIG_TEMP_START 0x88

#define BME280_REGISTER_DIG_PRES_START 0x8E

#define BME280_REGISTER_DIG_HUM_START  0xE1

//function to read from bme registers using spi

void BME_Read(u8 Add, u8 * buffer, u8 N){

u8 Write[N+1];

Write[0] = Add | 0x80;			//R/W = 1

XSpi_Transfer(&Spi, Write, buffer, N+1);

}

//function to write in bme registers using spi

void BME_Write(u8 Add, u8 val){

u8 Write[2], Read[2];

Write[0] = Add & 0x7F;		//R/W = 0

Write[1] = val;

XSpi_Transfer(&Spi, Write, Read, 2);

}

void BME_Wait(void) {

u8 Read[2];

do{

BME_Read(0xF3, Read, 1 );

} while (Read[1] != 0);

}

//declaring global variables

struct GLOBALS {

int32_t t_fine;

int32_t read_dig_t1, read_dig_t2, read_dig_t3;

int64_t read_dig_p1, read_dig_p2, read_dig_p3, read_dig_p5, read_dig_p4, read_dig_p6, read_dig_p7, read_dig_p8, read_dig_p9;

int32_t read_dig_h1, read_dig_h2, read_dig_h3, read_dig_h5, read_dig_h4, read_dig_h6;

u8 readregisters[10];

} g;

//function to read trimming parameters

void trimming_readout(){

u8 readtemp[10];

u8 readpress[20];

u8 readhumid[10];

u8 readhumidA1[2];

BME_Read(BME280_REGISTER_DIG_TEMP_START,readtemp,6);

g.read_dig_t1 = (unsigned short)(readtemp[1] + (u16)readtemp[2]*256);

g.read_dig_t2 = (signed short)(readtemp[3] + (u16)readtemp[4]*256);

g.read_dig_t3 = (signed short)(readtemp[5] + (u16)readtemp[6]*256);

BME_Read(BME280_REGISTER_DIG_PRES_START,readpress,18);

g.read_dig_p1 = (unsigned short)(readpress[1] + (u16)readpress[2]*256);

g.read_dig_p2 = (signed short)(readpress[3] + (u16)readpress[4]*256);

g.read_dig_p3 = (signed short)(readpress[5] + (u16)readpress[6]*256);

g.read_dig_p4 = (signed short)(readpress[7] + (u16)readpress[8]*256);

g.read_dig_p5 = (signed short)(readpress[9] + (u16)readpress[10]*256);

g.read_dig_p6 = (signed short)(readpress[11] + (u16)readpress[12]*256);

g.read_dig_p7 = (signed short)(readpress[13] + (u16)readpress[14]*256);

g.read_dig_p8 = (signed short)(readpress[15] + (u16)readpress[16]*256);

g.read_dig_p9 = (signed short)(readpress[17] + (u16)readpress[18]*256);

BME_Read(0xA1,readhumidA1,1);

BME_Read(BME280_REGISTER_DIG_HUM_START,readhumid,7);

g.read_dig_h1 = (unsigned char)readhumidA1[1];

g.read_dig_h2 = (signed short)(readhumid[1] + (u16)readhumid[2]*256);

g.read_dig_h3 = (unsigned char)readhumid[3];

g.read_dig_h4 = (signed short)((readhumid[5] & 0x0F) + (u16)readhumid[4]*16);

g.read_dig_h5 = (signed short)(((readhumid[5] & 0xF0)>>4) + ((u16)readhumid[6]*16));

g.read_dig_h6 = (signed char)readhumid[7];

}

int main()

{

init_platform();

spiInit();

BME_Write(0xE0, 0xB6); //Reset the Device

BME_Wait();

trimming_readout();

while(1){

readBuff();

float temp1=readbmetemp();    //Reading temperature

for(int i=0; i<10;i++){};

float press=readbmepress();   //Reading pressure

for(int i=0; i<10;i++){};

float humid = readbmehumid();  //Reading humidity

for(int i=0; i<10;i++){};

}

cleanup_platform();

return 0;

}

//function to initialise spi

int spiInit(void){

int Status;

ConfigPtr = XSpi_LookupConfig(0U);

if (ConfigPtr == NULL) {

return XST_DEVICE_NOT_FOUND;

}

Status = XSpi_CfgInitialize(&Spi, ConfigPtr,ConfigPtr->BaseAddress);

if (Status != XST_SUCCESS) {

return XST_FAILURE;

}

Status = XSpi_SetOptions(&Spi, XSP_MASTER_OPTION );

if (Status != XST_SUCCESS) {

return XST_FAILURE;

}

XSpi_Start(&Spi);

XSpi_IntrGlobalDisable(&Spi);

XSpi_SetSlaveSelect(&Spi, 0x01);

}

void readBuff(){

BME_Write(0xF2, 0x02);	//Calculate only humidity

BME_Write(0xF4, 0x45);	//Calculate only temperature in Forced Mode

BME_Read(0xF7,g.readregisters,8);

}

//function to calculate temperature

float readbmetemp(){

int32_t tvar0, tvar1=0;

int32_t tvar2=0;

int32_t read_adc = {(g.readregisters[6]>>4) + (int32_t)g.readregisters[5] * 16 + (int32_t)g.readregisters[4] * 256 * 16};

tvar0 = (read_adc>>3)-(g.read_dig_t1<<1);

tvar1 = (tvar0*g.read_dig_t2)>>11;

tvar2 = ((((tvar0 >> 1)*(tvar0 >> 1))>>12)*(g.read_dig_t3))>>14;

g.t_fine = tvar1 + tvar2 ;

int32_t T = (g.t_fine * 5 + 128) / 256;

return (float) T/100.0;

}

//function to calculate pressure

float readbmepress(){

int64_t pvar1=0;

int64_t pvar2=0;

int64_t pvar3=0;

int64_t pvar4=0;

int32_t adc_p = {(g.readregisters[3]>>4) + (int32_t)g.readregisters[2] * 16 + (int32_t)g.readregisters[1] * 256 * 16};

pvar1 = ((int64_t)g.t_fine) - 128000;

pvar2 = pvar1 * pvar1 * (int64_t)g.read_dig_p6;

pvar2 = pvar2 + ((pvar1 * (int64_t)g.read_dig_p5) * 131072);

pvar2 = pvar2 + (((int64_t)g.read_dig_p4) * 34359738368);

pvar1 = ((pvar1 * pvar1 * (int64_t)g.read_dig_p3) / 256) + ((pvar1 * ((int64_t)g.read_dig_p2) * 4096));

pvar3 = ((int64_t)1) * 140737488355328;

pvar1 = (pvar3 + pvar1) * ((int64_t)g.read_dig_p1) / 8589934592;

if (pvar1 == 0) {

return 0; // avoid exception caused by division by zero

}

adc_p <<= 4;

pvar4 = 1048576 - adc_p;

pvar4 = (((pvar4 * 2147483648) - pvar2) * 3125) / pvar1;

pvar1 = (((int64_t)g.read_dig_p9) * (pvar4 / 8192) * (pvar4 / 8192)) / 33554432;

pvar2 = (((int64_t)g.read_dig_p8) * pvar4) / 524288;

pvar4 = ((pvar4 + pvar1 + pvar2) / 256) + (((int64_t)g.read_dig_p7) * 16);

float P = pvar4 / 256.0;

//float P = (uint32_t)(((pvar4 / 2) * 100) / 128);

return P;

}

//function to calculate humidity

float readbmehumid(){

int32_t hvar1, hvar2, hvar3, hvar4, hvar5;

int32_t adc_h = {(g.readregisters[8]) + (int32_t)g.readregisters[7] * 16 *16};

hvar1 = g.t_fine - ((int32_t)76800);

hvar2 = (int32_t)(adc_h * 16384);

hvar3 = (int32_t)(((int32_t)g.read_dig_h4) * 1048576);

hvar4 = ((int32_t)g.read_dig_h5) * hvar1;

hvar5 = (((hvar2 - hvar3) - hvar4) + (int32_t)16384) / 32768;

hvar2 = (hvar1 * ((int32_t)g.read_dig_h6)) / 1024;

hvar3 = (hvar1 * ((int32_t)g.read_dig_h3)) / 2048;

hvar4 = ((hvar2 * (hvar3 + (int32_t)32768)) / 1024) + (int32_t)2097152;

hvar2 = ((hvar4 * ((int32_t)g.read_dig_h2)) + 8192) / 16384;

hvar3 = hvar5 * hvar2;

hvar4 = ((hvar3 / 32768) * (hvar3 / 32768)) / 128;

hvar5 = hvar3 - ((hvar4 * ((int32_t)g.read_dig_h1)) / 16);

hvar5 = (hvar5 < 0 ? 0 : hvar5);

hvar5 = (hvar5 > 419430400 ? 419430400 : hvar5);

uint32_t H = (uint32_t)(hvar5 / 4096);

return (float)H / 1024.0;

}